CE3001 Advanced Computer Architecture

Lab4 Report

Yang Yubei

U1820525L

SEP1

- Lab Questions

Write the MIPS assembly code for the computation of “e = (a-b)*(c^d)”. Note that all variables are integers. The addresses and data of the variables are given in table below. You can load this data to data memory.

- Write the MIPS assembly code for the computation of “e = (a-b)*(c^d)” with minimum number of instructions.

The minimum number of instructions: 8

Analysis of Data dependencies

| No.Inst | Instruction | Clock Cycle |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 1 | LW $1, 1($0) | F | D | E | M | W |  |  |  |  |  |  |  |
| 2 | LW $2, 2($0) |  | F | D | E | M | W |  |  |  |  |  |  |
| 3 | SUB $3, $1, $2 |  |  | F | D | E | M | W |  |  |  |  |  |
| 4 | LW $4, 3($0) |  |  |  | F | D | E | M | W |  |  |  |  |
| 5 | LW $5, 4($0) |  |  |  |  | F | D | E | M | W |  |  |  |
| 6 | XOR $6, $4, $5 |  |  |  |  |  | F | D | E | M | W |  |  |
| 7 | MUL $7, $3, $6 |  |  |  |  |  |  | F | D | E | M | W |  |
| 8 | SW $7, 5($0) |  |  |  |  |  |  |  | F | D | E | M | W |

- Cycle 4:

At decode stage of Inst3, values from memory is not yet stored in $1 and $2.

- Cycle 7:

At decode stage of Inst6, values from memory is not yet stored in $4 and $5.

- Cycle 8:

At decode stage of Inst7, value of $4^$5 is not yet written back to $6.

- Cycle 9:

At decode stage of Inst8, value of $3*$6 is not yet written back to $7.

We need add NOP instructions to eliminate data dependencies.

- Modify the MIPS assembly code for the computation of “e = (a-b)*(c^d)” for a five stage pipelined architecture given in lab 4, after including NOPs for removing data-dependencies.

Without Data dependencies

| No.Inst | Instruction | Clock Cycle |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| 1 | LW $1, 1($0) | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | LW $2, 2($0) |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | NOP |  |  | S |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | NOP |  |  |  | S |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | SUB $3, $1, $2 |  |  |  |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |
| 6 | LW $4, 3($0) |  |  |  |  |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |
| 7 | LW $5, 4($0) |  |  |  |  |  |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |
| 8 | NOP |  |  |  |  |  |  |  | S |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | NOP |  |  |  |  |  |  |  |  | S |  |  |  |  |  |  |  |  |  |  |  |
| 10 | XOR $6, $4, $5 |  |  |  |  |  |  |  |  |  | F | D | E | M | W |  |  |  |  |  |  |
| 11 | NOP |  |  |  |  |  |  |  |  |  |  | S |  |  |  |  |  |  |  |  |  |
| 12 | NOP |  |  |  |  |  |  |  |  |  |  |  | S |  |  |  |  |  |  |  |  |
| 13 | MUL $7, $3, $6 |  |  |  |  |  |  |  |  |  |  |  |  | F | D | E | M | W |  |  |  |
| 14 | NOP |  |  |  |  |  |  |  |  |  |  |  |  |  | S |  |  |  |  |  |  |
| 15 | NOP |  |  |  |  |  |  |  |  |  |  |  |  |  |  | S |  |  |  |  |  |
| 16 | SW $7, 5($0) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | D | E | M | W |

The number of instructions: 16

Clock Cycle : 20

We can reorder then instruction to make it more efficient.

Minimum Clock Cycle & Without Data dependencies

| No.Inst | Instruction | Clock Cycle |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| 1 | LW $1, 1($0) | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | LW $2, 2($0) |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |
| 3 | LW $4, 3($0) |  |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |
| 4 | LW $5, 4($0) |  |  |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |
| 5 | SUB $3, $1, $2 |  |  |  |  | F | D | E | M | W |  |  |  |  |  |  |  |  |
| 6 | NOP |  |  |  |  |  | S |  |  |  |  |  |  |  |  |  |  |  |
| 7 | XOR $6, $4, $5 |  |  |  |  |  |  | F | D | E | M | W |  |  |  |  |  |  |
| 8 | NOP |  |  |  |  |  |  |  | S |  |  |  |  |  |  |  |  |  |
| 9 | NOP |  |  |  |  |  |  |  |  | S |  |  |  |  |  |  |  |  |
| 10 | MUL $7, $3, $6 |  |  |  |  |  |  |  |  |  | F | D | E | M | W |  |  |  |
| 11 | NOP |  |  |  |  |  |  |  |  |  |  | S |  |  |  |  |  |  |
| 12 | NOP |  |  |  |  |  |  |  |  |  |  |  | S |  |  |  |  |  |
| 13 | SW $7, 5($0) |  |  |  |  |  |  |  |  |  |  |  |  | F | D | E | M | W |

Minimum number of instructions: 13

Minimum Clock Cycle : 17

- Show the snapshot of instruction and data-memory (all values in hexadecimal) from the ISIM simulation window

Data is stored in text file ‘dmem_txt.txt’

First column: address; Second column: data value

| dmem_test.txt | ISIM simulation window |
| --- | --- |

This is Initialize txt.                      This is data memory after execution.

The set of instructions is stored in a text file ‘imem_test.txt’.

Note: An extra NOP is needed at the beginning of instruction

| imem_test.txt | ISIM simulation window |
| --- | --- |

The left graph shows the full execution in a 5-stage pipeline architecture without data forwarding.

The right graph shows the register memory after execution.

Further illustration will be in next part on LW, SW instruction and execution.

- Explain the working of the five-stage pipeline both for LW and SW instruction (used in this code) using ISIM window as reference.

- LW instruction

Take 1C010001 //LW $1, 1($0) as an example for illustration.

The addressing of LW (I-type instruction)

| opcode | rs | rt | immediate |
| --- | --- | --- | --- |
|  | $0 | $1 | 1 |
| 000111 | 00000 | 00001 | 0000 0000 0000 0001 |

- Fetch

Instruction memory of address 0x01 is fetched to INST[31:0] with value of 0x1C010001

- Decode

- Address of $1 is write register address, stored in pipeline register waddr_ID_EXE[4:0].

- $0 is read from register file, pass value 0 to rdata1, then is stored in pipeline register : rdata1_ID_EXE[31:0].

- Immediate offset, 1, is sign extended and store in pipeline register imm_ID_EXE[31:0].

- Execute

- aluop_ID_EXE[2:0] = 000 indicates an ADD operation.

- alusrc_ID_EXE is set to logic 1 indicate a LW instruction, which ALU should retrieve and add up imm_ID_EXE[31:0] and rdata1_ID_EXE[31:0].

- aluout[31:0] = imm_ID_EXE[31:0] + rdata1_ID_EXE[31:0]= 1.

- aluout[31:0] is passed to pipeline register aluout_EXE_MEM[31:0] = 1.

waddr_ID_EXE[4:0] is passed to waddr_EXE_MEM[4:0].

- Memory

- aluout_EXE_MEM[31:0] = 1 indicates read from data memory of address 0x0001.

- MemWrite is logic 0 indicates no writing to memory in LW instruction.

- waddr_EXE_MEM[4:0] is passed to waddr_MEM_WB[4:0].

- Write Back

- The value retrieved from address 0x01 of data memory is 0x0D. (In diagram is 13 since decimal radix is used.) This value is straightly output to dmemdata[31:0].

- MemtoReg is set to logic 0. Hence, wdata_wb gets data from dmemdata[31:0] (not aluout_MEM_WB).

- WriteEnable is set to logic 1.

waddr_MEM_WB[4:0] = 1 and wdata_wb is written to $1.

The below graph is a snapshot of LW example.

- SW instruction

Take 20070005 //SW $7, 5($0) as an example for illustration.

The addressing of SW (I-type instruction)

| opcode | rs | rt | immediate |
| --- | --- | --- | --- |
|  | $0 | $7 | 5 |
| 001000 | 00 000 | 0 0111 | 0000 0000 0000 0101 |

- Fetch Instruction memory of address 0x0D is fetched to INST[31:0] with value of 0x20070005.

- Decode

- $0 is read from register file, pass value 0 to rdata1, then is stored in pipeline register: rdata1_ID_EXE[31:0].

- $7 is read from register file, pass value 70(decimal) to rdata2, then is stored in pipeline register: rdata2_ID_EXE[31:0].

- Immediate offset, 5, is sign extended and store in pipeline register imm_ID_EXE[31:0].

- Execute

- aluop_ID_EXE[2:0] = 000 indicates an ADD operation.

- alusrc_ID_EXE is set to logic 1 indicate a LW instruction, which ALU should retrieve and add up imm_ID_EXE[31:0] and rdata1_ID_EXE[31:0].

aluout[31:0] = imm_ID_EXE[31:0] + rdata1_ID_EXE[31:0]= 5.

- aluout[31:0] is passed to pipeline register aluout_EXE_MEM[31:0] = 5.

Value of rdata2_ID_EXE[31:0] is passed to rdata2_EXE_MEM[31:0]

- Memory

- aluout_EXE_MEM[31:0] = 5 indicates write to data memory of address 0x0005.

- MemWrite is logic 1 indicates writing to memory in SW instruction.

- rdata2_EXE_MEM[31:0] is written to data memory of address 0x0005.

- Write Back

This stage is not skipped but there is no write back to register file in SW instruction. The WriteEnable is set to logic 0.

- Indicate the execution time for running this program along with a snapshot of starting and ending time of the code in the ISIM simulator.

From the above snapshot, the execution time is 510.500 ns.

The formula is:

Execution time = Clock Period * CPI * No.of Instructions

Clock period is 30ns. Then, Execution time = 30ns * 1 * 17 = 510 ns.

The cursor measurement is close to what we expected.

- Calculate the steady state CPI of the code while running in a five-stage pipelined architecture.

CPI=no.of Inst+no.of Stallsno.of Inst=8+513=1.625